Enhancement/Customization: E3 Export Bundle Segment Information for COMSOL

Customer: Trend Industries

Objective:

Provide the harness team with the ability to export bundle segment data on a selected harness segment, consisting of the outer and inner diameter of the cable and individual wires.  The exported data shall consist of an Excel spreadsheet of the bundle segment.

Background:

The harness group is looking to use Zuken’s ability of displaying the minimum cross-section in order to import an Excel formatted file of a bundle segment cross-section into the COMSOL simulation tool in order to perform a thermal analysis.

E3.HarnessAnalyzer:

User selects segment:

With pressing the “Export Comsol” – button the excel file is generated for the selected bundle in selected configuration (if there are multiple configuration).

Note: The graphical representation of the bundle segment is generated on-the-fly via an algorithm.  It is not user modifiable.

Detailed Requirements:

The development and implementation of this enhancement shall be divided into multiple-phases.

Note: Phase 2 is not committed yet.  It will be reevaluated after Phase 1 completion.

Preliminary

- Provide a manually created Excel output to serve as a mock up, containing the X-axis and Y-axis for the circle for the inner and outer diameter of each wire.  See Requirement #3 in Phase 1 below for format.

Phase 1

User must manually select the desired segment within E3.HarnessAnalyzer

- Excel: Export an Excel document listing the radius, center x-axis, center y-axis values of the inner and outer diameter of each wire.

*The Wire ID column value shall be composed of the <Cable Device Designation>:<Wire Name> + ID or OD string

  - Example:

    - W350:0532-0.50 mm²-Black ID

    - W350:0532-0.50 mm²-Black OD

| Column | Value |
| --- | --- |
| Wire ID | List the inner and outer diameter per each wire ID in individual rows. *The wire ID must match the same on the Zuken E3.series side.* |
| Radius | Radius of circle |
| Center-X | Center X-axis coordinate of circle |
| Center-Y | Center Y-axis coordinate of circle |
| Signal Name | Net Name/Signal Name |

- The file name for the Excel and DXF outputs shall consist of the following naming structure:

    - <harness_filename>--segment_ID_<Segment name>.xlsx

    - <harness_filename>-segment_ID_<Segment name>.dxf

    - <harness_filename> refers to the HCV file name

      - Note: User is responsible for naming the HCV file*

    - <Segment name> refers to the Segment ID generated by E3.HarnessAnalayzer.

Example:

10-400-000123-A0-segment_ID_BNS3296380.xlsx

Example: Original Excel Output required for COMSOL import

Example: E3 wire naming with Cable Device Designation

Example view of COMSOL import of E3.HarnessAnalzyer Excel

- Develop a method for automating the naming and save path of exporting the HCV file via the E3.3DRoutingBridge

  - User selects the active formboard sheet and runs the automated HCV method script

  - File name: <Drawing Number>.HCV

    - Where the attribute <Drawing Number> is derived from the active formboard sheet properties

  - Provide a configurable export save path.  The path shall be generated with the following parameters:

    - \\<Server Path>\<E3 Project Name>\<Drawing Number>\

      - Where the <E3 Project Name> value is derived from the E3 Project Name

      - Where the attribute <Drawing Number> is derived from the active formboard sheet properties

Phase 2

Important: as of April 15th, 2020 it is not possible to commit to the implementation of Phase 2.  Zuken requires the harness team to verify that the results from Phase 1 meets expectations and can be loaded into COMSOL.

- Enhance the export capability to provide a batch export of every segment within the harness:

- Excel

  - Each file name will consist:

    - <harness_filename>--segment_ID_<Segment name>.xlsx

Example:

10-400-000123-A0-segment_ID_BNS3296380.xlsx

- Provide a configurable setting for the batch export to save the files to a definable file directory on the network share.

  - Generate a folder per harness file name

    - Example: If the HCV file name is called 10-400-000123-A0.hcv

      - Generate a folder called <network path>\10-400-000123-A0\

        - Contents inside this folder:

          - 10-400-000123-A0-segment_ID_BNS3296380.xlsx

          - 10-400-000123-A0-segment_ID_BNS3296382.xlsx

- Provide a method for searching the Segment ID within E3.HarnessAnalyzer for troubleshooting purposes

  - Match Segment ID with Branch ID from E3.formboard.

  - Use case is if user is reviewing the Excel spreadsheet and needs to quickly determine which branch this references to.

          - 10-400-000123-A0-segment_ID_BNS3296380.xlsx quickly be able to reference where ID_BNS3296380 is within the formboard drawing.

Acceptance Criteria:

Provide the Excel outputs of the selected bundle segments to harness team to import into COMSOL and test.